http://www.valu-bilt.com/cgi-bin/sgin0101.exe?UID=2003021322465818&GEN2=&GEN3=&GEN9=&T1=A+595074&FNM=00&UREQB=2&UREQD=6

Front switch positions                rear posts connected

1				none (Off)

- (1 & 4) Accessories

- (1,2,4) Lights and instrument cluster (Run)

- (1 & 3) Heat Glow Plugs (spring return to Run)

- (1,3,5) Engage Starter (spring return to Run)

Rear connection posts		purpose

- accepts 12v via ammeter wire #5

- sends 12v to fuse panel via wire #6

- sends 12v to glow plugs via wire #12

- available 12v for fused accessories

- sends 12v to starter solenoid via wire #10

the rear posts are unmarked except for #5. The diagram below arbitrarily designates posts #1 through #4 clockwise from #5.